Project Summary: ThingSpeak Weather Dashboard using OpenWeather API

Objective

The goal of this project was to build a live weather monitoring dashboard in ThingSpeak, similar to the weather dashboard previously created in Node-RED.

The dashboard fetches real-time weather data from the OpenWeather API, stores it in a ThingSpeak channel, and displays it using gauges, numeric displays, and charts.

Technologies Used

- ThingSpeak (IoT cloud platform)

- MATLAB Analysis (built into ThingSpeak)

- OpenWeather API

- HTTP GET requests

- ThingSpeak Widgets

Data Collected

The project retrieves the following weather parameters:

| Parameter | ThingSpeak Field |
| --- | --- |
| Temperature (°C) | Field 1 |
| Humidity (%) | Field 2 |
| Weather Code | Field 3 |
| Wind Speed (m/s) | Field 4 |
| Pressure (hPa) | Field 5 |

The actual weather text (e.g., Clouds, Rain, Clear) is converted into a numeric code before storing.

Weather Code Mapping

| Code | Weather Condition |
| --- | --- |
| 1 | Clear |
| 2 | Clouds |
| 3 | Rain |
| 4 | Drizzle |
| 5 | Thunderstorm |
| 6 | Snow |
| 0 | Unknown |

Working of the System

OpenWeather API

↓

MATLAB webread()

↓

Extract JSON data

↓

Convert weather text → weather code

↓

thingSpeakWrite()

↓

ThingSpeak Channel

↓

Dashboard Widgets & Charts

MATLAB Script Functions

The MATLAB script performs the following operations:

- Sends an HTTP request to OpenWeather.

- Reads the JSON response.

- Extracts:

  - temperature

  - humidity

  - pressure

  - wind speed

  - weather condition

- Converts the weather condition into a numeric code.

- Writes all values into the ThingSpeak channel.

Dashboard Components

Widgets Added

| Widget | Purpose |
| --- | --- |
| Temperature Gauge | Displays temperature |
| Humidity Gauge | Displays humidity |
| Wind Speed Gauge | Displays wind speed |
| Pressure Gauge | Displays pressure |
| Numeric Display | Displays weather code |

Automatic Charts

ThingSpeak automatically generates line charts for:

- Temperature

- Humidity

- Wind Speed

- Pressure

Automation

Using TimeControl, the MATLAB script can be executed automatically every few minutes, allowing the dashboard to update continuously without manual intervention.

Key Learning Outcomes

This project helped in learning:

- How IoT cloud platforms store sensor data

- How REST APIs work

- HTTP requests in MATLAB

- JSON data parsing

- ThingSpeak channel configuration

- Data visualization using gauges and charts

- Scheduling automatic cloud-based tasks

Final Result

A complete real-time weather dashboard was created in ThingSpeak that:

- Fetches live weather data from the internet

- Updates automatically

- Stores historical weather data

- Displays current conditions through gauges and charts

- Provides trends over time similar to a professional IoT monitoring dashboard

This project demonstrates the integration of web APIs, cloud IoT platforms, MATLAB scripting, and data visualization, making it a strong beginner-to-intermediate IoT and embedded systems project.